ENGR 3323: Signals and Systems

HW 9_Ch6

Q1) For the periodic signals x(t) and y(t) shown below:

- Find the exponential Fourier series for x(t) and y(t).

- Sketch the amplitude and phase spectra for signal x(t).

- Use Parseval’s theorem to approximate the power of the periodic signal x(t) by calculating the power of the first Nth harmonics, such that the strength of the Nth harmonic is 10% or more of the power of the DC component.

Q2) The exponential Fourier series of a certain function is given as

x(t) = (2 + j2)e-j3t + j2e-jt + 3 - j2ejt + (2 - j2)ej3t

- Sketch the exponential Fourier spectra.

- By inspection of the spectra in part (a), sketch the trigonometric Fourier spectra for x(t). Find the compact

trigonometric Fourier series from these spectra.

- Show that the trigonometric series found in part (b) is equivalent to the exponential series for x(t).

- Find the signal bandwidth.

Q3) Figure below shows the exponential Fourier spectra of a periodic signal x(t).

- By inspection of the Figure find the exponential Fourier series representing x(t).

- By inspection of the Figure, sketch the trigonometric Fourier spectra for x(t).

- By inspection of the trigonometric Fourier spectra found in part (b), find the trigonometric Fourier series for x(t).

- Show that the series found in parts (a) and (c) are equivalent.

Q4)Find the response of an LTIC system with transfer function  Hs=ss2+2s+3  to the periodic input

x(t) = (2 + j2)e-j3t + j2e-jt + 3 - j2ejt + (2 - j2)ej3t

Q5) Find the exponential Fourier series for a periodic signal x(t) shown in Figure below

The signal x(t) is applied at the input of an LTIC system shown above. Find the expression for the

output y(t).