Electrical Science Formula Sheet

i = dq/dt       Q=0tidt       vab = dw/dq       p = dw/dt = vi	w=totPdt	v = iR	   G=1/R        p = vi = i2R = v2/R

n=1Nin=0 	n=1Nvn=0 	Series: Req = R1 + R2 + … + Rn		Parallel: 1/Req = 1/R1 + 1/R2 + … + 1/Rn

vn = (Rn*v)/(R1+R2+…+RN)	q = Cv		C=ɛA/d	i = C dv/dt	v=1Ctotidt+v(to)    w=Cv2/2

Parallel: Ceq = C1 + C2 + …+Cn	Series:  1/Ceq = 1/C1 + 1/C2 + …+ 1/Cn	v = L di/dt  	L = N2μA/l

i=1Ltotv(t)dt+i(to)      w=Li2/2	Series: Leq = L1 + L2 + …+ Ln   Parallel:  1/Leq = 1/L1 + 1/L2 + …+ 1/Ln

Capacitor discharge: v(t) = V0e-t/τ    τ = RC	Inductor discharge: i(t) = I0e-t/τ    τ = L/R

Capacitor step response v(t) = v(ꝏ) + [v(0+) – v(ꝏ)]e-t/τ

Inductor step response i(t) = i(ꝏ) + [i(0+) – i(ꝏ)]e-t/τ

Complex Variables: z = x +jy		z = rejθ       r=x2+y2	θ=tan-1(yx)	    z* = x-jy          z* = re-jθ

z1z2 = r1 r2ej(θ1+ θ2) 	z1/z2 = (r1 /r2)ej(θ1- θ1)  	impedance:  ZC = 1/jωC	ZL = jωL	ZR = R

Steps to determine the node voltages:

  - Select a node as the reference node.

  - Assign voltages v1,v2,…,vn-1 to the remaining n-1 nodes. The voltages are referenced with respect to the reference node.

  - Apply KCL to each of the n-1 non-reference nodes. Use Ohm’s law to express the branch currents in terms of node voltages.

  - Solve the resulting simultaneous equations to obtain the unknown node voltages.

Steps to determine the node voltages with voltage source between nodes:

- A super-node is formed by enclosing a (dependent or independent) voltage source connected between two non-reference nodes and any elements connected in parallel with it.

- Take off all voltage sources in super-nodes and apply KCL to super-nodes.

- Put voltage sources back to the nodes and apply KVL to relative loops.

Steps to determine the mesh currents:

  - Assign mesh currents i1, i2, …, in to the n meshes.

  - Apply KVL to each of the n meshes. Use Ohm’s law to express the voltages in terms of the mesh currents.

  - Solve the resulting n simultaneous equations   to get the mesh currents.

Steps to determine the mesh currents with current source:

A super-mesh results when two meshes have a (dependent or independent) current source in common.

1- We create a super-mesh by excluding the current source and any elements connected in series with it.

2- Write KVL in the super-mesh using the loop currents of the two adjacent meshes.

3- The sum or the difference of the two mesh currents equals the current source in the common branch between the two meshes.

Thevenin’s Theorem

It states that a linear two-terminal circuit can be replaced by an equivalent circuit consisting of a voltage source VTH in series with a resistor RTH,

- VTH is the open-circuit voltage at the terminals.

- RTH is the input or equivalent resistance at the terminals when the independent sources are turned off.

- Independent sources are turned off by replacing a Voltage source by a short circuit and current source by open circuit.

Norton’s Theorem

It states that a linear two-terminal circuit can be replaced by an equivalent circuit of a current source IN in parallel with a resistor RN.

- IN is the short circuit current through the terminals.

- RN is the input or equivalent resistance at the terminals when the independent sources are turned off.

- Independent sources are turned off by replacing a Voltage source by a short circuit and current source by open circuit.

Maximum Power Transfer

For maximum Power transfer load resistor should equal Thevenin’s resistor

Capacitor

Inductor